| GROUNDS See Note (1) | FREQUENCY See Note (2) | INSPECTION/ MAINTENANCE PROCEDURES | WHO (Trade/ Volunteer) | NOTES |
| --- | --- | --- | --- | --- |
| Playground Equipment | Monthly/ Annually | Inspect for loose, cracked parts, sharp edges, rot & fall protection. |  |  |
| Planting | Monthly/ Annually | Inspect for plant and tree growth against building. |  |  |
| Concrete Patios & Retaining Walls | Quarterly | Inspect for cracks, shifting, water damage and drainage. |  |  |
| Sidewalks and Driveways | Monthly | Inspect for cracks and trip hazards. |  |  |
| BUILDING EXTERIOR See Note (3) | FREQUENCY | INSPECTION/ MAINTENANCE PROCEDURES | WHO IS DOING IT? |  |
| Crawlspaces, concrete Foundations | Annually/ ongoing | Inspect for dampness, vermin, inspect beams and posts for rot. Inspect foundations for cracks, shifting, and water damage. |  |  |
| Masonry Veneer | Annually | Inspect for efflorescence (white powder), inspect sealants. |  |  |
| Porches and Balconies | Annually/ ongoing | Clean and inspect for cracks, rot, trip hazards, secure railings & steps. |  |  |
| Wood Siding, Fascias & Trim | Annually/ Monthly | Inspect for cracks, water damage, deteriorated finishes and vermin. |  |  |
| Attic Roof Sheathing | Annually | Inspect for water damage, mildew and condensation. |  |  |
| Attic Insulation | Annually | Inspect for moisture, vermin, voids, ensure insulation dry |  |  |
| Attic Ventilation | Annually | Inspect for obstructions, rot, vermin. |  |  |
| Vinyl Decks | Annually | Check for lifting, water damage, damaged flashing. |  |  |
| Concealed Waterproofing (parkade) | Quarterly | Check underside of slabs for evidence of leaks. Pressure Clean annually |  |  |
| Stucco | Quarterly | Check for signs of water leakage, mold, mildew and staining, cracks, caulking |  |  |
| Roof Shingles | Quarterly | Check for curled or missing shingles, excessive moss, damaged flashing |  |  |
| Gutters/ Downspouts | Semi Annual | Clean inside and out, repair as needed, check for loose downspouts and brackets, check outlets for obstructions |  |  |
| Vinyl Siding | Annually | Inspect for damaged siding and water damage. |  |  |
| Steel Siding | Annually | Inspect for water damage and damaged sheets. |  |  |
| BUILDING EXTERIOR See Note (3) | FREQUENCY | INSPECTION/ MAINTENANCE PROCEDURES | WHO IS DOING IT? |  |
| Flat Roofs | Quarterly/ Monthly | Inspect for ponding, loose flashing, plugged drains, air pockets, blisters & debris. |  |  |
| Roof Hatch | Monthly | Inspect for forced entry, hardware operation, leaks and weather stripping. |  |  |
| Metal Doors and Frames | Annual/ Ongoing | Inspect for forced entry, correct hardware operation, weather stripping and sealants, thresholds and warping |  |  |
| Wood Doors and Frames | Annual/ Ongoing | Inspect for forced entry, correct hardware operation, weather stripping and sealants. |  |  |
| Automatic Door Openers | Monthly | Inspect for damage, forced entry, wear, and test all safety features. |  |  |
| Overhead Parkade Doors | Quarterly | Inspect for proper operation and damage to components, test safety features. |  |  |
| Aluminum Entrances and Storefront | Annually/ Ongoing | Inspect for forced entry, hardware operation, closer adjustment. |  |  |
| Windows and Doors | Annually | Inspect for leaks, hardware operation & sealing. |  |  |
| Hose Bibs | Quarterly | Inspect for damage and winterize in advance of freezing temperatures. |  |  |
| Outdoor Outlets | Monthly | Test ground fault plugs. |  |  |
| Dryer Vents | Annually | Clean bird screens monthly and vents annually (recommended). |  |  |
| Floor and Trench Drains | Annually | Clean out drains and trenches. |  |  |
| Chimneys | Annually | Clean and check for cracks, loose bricks, leaks or damage. Include fireplaces and wood stoves. |  |  |
| BUILDING INTERIOR & SERVICES See Note (4) | FREQUENCY | INSPECTION/ MAINTENANCE PROCEDURES | WHO IS DOING IT? |  |
| Door Handles, Hinges and Closures | Monthly/ ongoing | Check hardware for proper function, check and test fire exit hardware. |  |  |
| Residential Appliances | Annually | Check fridge seals and drain tubes. Check stove burners and wires for electrical shorting or grease build up. Check hood fans for filters, venting, grease build up and fire hazards. |  |  |
| Walls and Ceilings | Annually/ ongoing | Inspect for mold, water damage, holes, cracking, paint and humidity levels. |  |  |
| BUILDING INTERIOR & SERVICES See Note (4) | FREQUENCY | INSPECTION/ MAINTENANCE PROCEDURES | WHO IS DOING IT? |  |
| Elevators | Monthly | Have an elevator contractor service and maintain the elevator equipment. |  |  |
| Boiler – Large Building | Monthly | Service monthly, including circulating pumps. Adjust for seasonal temperatures. |  |  |
| Boiler/Furnace – Small Building | Semi Annually or Quarterly | Service annually, change filter _ to _ times per year. Adjust for seasonal temps. |  |  |
| Air Conditioning | Seasonally | Inspect for damage, icing, noise & leaks. Clean condenser. |  |  |
| Ductwork | Annually | Clean and inspect for rust or damage. |  |  |
| Gas Piping | Annually | Inspect for leaks and clearance between gas fired fixtures and combustibles. |  |  |
| Hot Water Tank | Annually | Open drain, inspect pressure reducer valve, check for leaks. |  |  |
| Electrical Panel and wiring | Annually/ Monthly | Inspect for loose, frayed, exposed or overloaded wires/plugs. Test ground fault plugs monthly. |  |  |
| Electrical Radiant Heater | Annually | Inspect for damage. |  |  |
| Fire Alarm Sys, Extinguishers, Hoses, Sprinklers, Heat & Smoke Detectors | Annually/ Ongoing | Inspect annually using a qualified inspection firm. Have staff conduct daily, weekly and monthly checks from Fire Safety Plan. |  |  |
| Emergency Generators | Weekly/ and quarterly or Semi-annually | Run emergency generators once per week for 20 mins., and have inspected by a qualified firm every 3-6 months. Maintain fuel supply. |  |  |

NOTES:

- Includes site improvements outside the building envelope.

- The reference section found in building operating and maintenance manual submitted by the contractor and or/ building envelope warranty or other warranties (as applicable).  Review any maintenance manuals to determine warranty requirements.

- Includes foundation walls, roofs, windows and doors.

- Includes building services and interior finishes.